3.1    Troubleshooting device security using IOS AAA

(TACACS+, RADIUS, local database)

| File | Location | Description |
| --- | --- | --- |
| 3.1 Troubleshoot device security using IOS AAA (TACACS+, RADIUS, local database).yaml | \ENARSI v1.1 Topics\3.0 Infrastructure Seurity\3.1 Troubleshoot device security using IOS AAA (TACACS+, RADIUS, local database) | YAML file. CML Topology file |
| Bash.sh | \ENARSI v1.1 Topics\3.0 Infrastructure Seurity\3.1 Troubleshoot device security using IOS AAA (TACACS+, RADIUS, local database)\Radius config files | Radius container startup script |
| Radius.conf | \ENARSI v1.1 Topics\3.0 Infrastructure Seurity\3.1 Troubleshoot device security using IOS AAA (TACACS+, RADIUS, local database)\Radius config files | Radius configuration |
| Users.conf | \ENARSI v1.1 Topics\3.0 Infrastructure Seurity\3.1 Troubleshoot device security using IOS AAA (TACACS+, RADIUS, local database)\Radius config files | Users on Radius server |

Topology

Files content

Bash.sh

# insert more commands here

ip address add dev eth0 192.168.2.254/24

ip link set dev eth0 up

ip route add default via 192.168.2.1 dev eth0

#

# keep the next line to indicate that the machine is ready

echo "READY" >/dev/console

exit 0

Radius.conf

# Basic freeRADIUS onfiguration

#

# This is what's included in the new container image for 2.9.1

# It's basically the stripped down version of the radiusd.conf that

# ships with the freeRADIUS distribution minus all the comments. Also:

# - no clients.conf dir inclusion (a wildcard client is defined below, note

#   the shared secret)

# - a few more lines/sections have been commented out that were found to not

#   be required for very basic RADIUS testing.

#

# NOTE it should be possible to successfully use this radiusd.conf with the

#      container image that ships with CML 2.9.0

prefix = /usr

exec_prefix = /usr

sysconfdir = /etc

localstatedir = /var

sbindir = ${exec_prefix}/sbin

logdir = /var/log/freeradius

raddbdir = /etc/freeradius/3.0

radacctdir = ${logdir}/radacct

name = freeradius

confdir = ${raddbdir}

modconfdir = ${confdir}/mods-config

certdir = ${confdir}/certs

cadir   = ${confdir}/certs

run_dir = ${localstatedir}/run/${name}

db_dir = ${raddbdir}

libdir = /usr/lib/freeradius

pidfile = ${run_dir}/${name}.pid

max_request_time = 30

cleanup_delay = 5

max_requests = 16384

hostname_lookups = no

log {

destination = syslog

colourise = yes

file = ${logdir}/radius.log

syslog_facility = daemon

stripped_names = no

auth = yes

auth_reject = yes

auth_badpass = yes

auth_goodpass = yes

msg_denied = "You are already logged in - access denied"

}

checkrad = ${sbindir}/checkrad

#ENV {

#}

security {

user = freerad

group = freerad

allow_core_dumps = no

max_attributes = 200

reject_delay = 1

status_server = yes

}

proxy_requests = no

#$INCLUDE proxy.conf

#$INCLUDE clients.conf

client all-devices {

ipaddr = *

secret = RadiusPwd

require_message_authenticator = no

nas_type = cisco

}

thread pool {

start_servers = 5

max_servers = 32

min_spare_servers = 3

max_spare_servers = 10

max_requests_per_server = 0

auto_limit_acct = no

}

modules {

$INCLUDE mods-enabled/

}

#instantiate {

#}

policy {

$INCLUDE policy.d/

}

$INCLUDE sites-enabled/

Users.conf

george Cleartext-Password := "geopwd"

Reply-Message = "Welcome, George!",

cisco-avpair = "shell:priv-lvl=15"

testuser Cleartext-Password := "testpassword"

Reply-Message = "Welcome, Test user",

cisco-avpair = "shell:priv-lvl=15"

pui Cleartext-Password := "puipwd"

Reply-Message = "Welcome, Pui!",

cisco-avpair = "shell:priv-lvl=15"

Node adaptations

On PCs – for ssh connection

Add this to etc/ssh/ssh_config

Host *

KexAlgorithms +diffie-hellman-group14-sha1,diffie-hellman-group-exchange-sha1

HostKeyAlgorithms +ssh-rsa

Output files – use tail commands to see live updates

On Radius check these files:

On Tacacs+ check these files:

Tac_plus.log

Tacplus-acct.log

Useful show / debug commands

debug tacacs

debug radius

debug aaa authentication

debug aaa authorization

debug aaa accouting

show tacacs public

show radius statistics

Output example

R2(config)#router eigrp 1

R2(config-router)#

*Oct  7 03:03:12.849: AAA/ACCT/3844(00000010): Pick method list 'T_ACCT_COMMANDS'

*Oct  7 03:03:12.849: AAA/ACCT/SETMLIST(00000010): Handle 6E000010, mlist 0E879058, Name T_ACCT_COMMANDS

*Oct  7 03:03:12.849: Getting session id for CMD(00000010) : db=D072908

*Oct  7 03:03:12.849: AAA/ACCT/CMD(00000010): add, count 3

*Oct  7 03:03:12.849: AAA/ACCT/EVENT/(00000010): COMMAND

*Oct  7 03:03:12.851: AAA/ACCT/CMD(00000010): Queueing record is COMMAND osr 1

*Oct  7 03:03:12.851: AAA/ACCT/CMD(00000010): free_rec, count 2

*Oct  7 03:03:12.851: AAA/ACCT/CMD(00000010): Setting session id 30 : db=D072908

*Oct  7 03:03:12.860: AAA/ACCT(00000010): Accounting method=TACACS_GROUP (TACACS+)

*Oct  7 03:03:12.860: TPLUS: Queuing AAA Accounting request 16 for processing

*Oct  7 03:03:12.864: TPLUS: processing accounting request id 16

*Oct  7 03:03:12.864: TPLUS: Sending AV task_id=29

*Oct  7 03:03:12.864: TPLUS: Sending AV timezone=UTC

*Oct  7 03:03:12.864: TPLUS: Sending AV service=shell

*Oct  7 03:03:12.864: TPLUS: Sending AV priv-lvl=15

*Oct  7 03:03:12.864: TPLUS: Sending AV cmd=router eigrp 1 <cr>

*Oct  7 03:03:12.864: TPLUS: Accounting request created for 16(george)

*Oct  7 03:03:12.864: TPLUS: using previously set server 192.168.1.254 from group TACACS_GROUP

*Oct  7 03:03:12.866: TPLUS(00000010)/0/NB_WAIT/D084EF0: Started 5 sec timeout

*Oct  7 03:03:12.871: TPLUS(00000010)/0/NB_WAIT: socket event 2

*Oct  7 03:03:12.872: TPLUS(00000010)/0/NB_WAIT: wrote entire 110 bytes request

*Oct  7 03:03:12.872: TPLUS(00000010)/0/READ: socket event 1

*Oct  7 03:03:12.872: TPLUS(00000010)/0/READ: Would block while reading

*Oct  7 03:03:12.876: TPLUS(00000010)/0/READ: socket event 1

*Oct  7 03:03:12.877: TPLUS(00000010)/0/READ: read entire 12 header bytes (expect 5 bytes data)

*Oct  7 03:03:12.877: TPLUS(00000010)/0/READ: socket event 1

*Oct  7 03:03:12.877: TPLUS(00000010)/0/READ: read entire 17 bytes response

*Oct  7 03:03:12.877: TPLUS(00000010)/0/D084EF0: Proces

R2(config-routsing the reply packet

*Oct  7 03:03:12.877: TPLUS: Received accounting response with status PASS

*Oct  7 03:03:12.878: AAA/ACCT/CMD(00000010): STOP protocol reply PASS

*Oct  7 03:03:12.878: AAA/ACCT(00000010): Accounting response status = SUCCESS

*Oct  7 03:03:12.878: AAA/ACCT(00000010): Send STOP accounting notification to EM successfully

*Oct  7 03:03:12.878: AAA/ACCT/CMD(00000010): Cleaning up from Callback osr 0

*Oct  7 03:03:12.879: AAA/ACCT/CMD(00000010) Record not present

*Oct  7 03:03:12.879: /AAA/ACCTCMD(00000010) reccnt 2, csr FALSE, osr 0er)#

R2(config-router)#no router eigrp 1

R2(config)#

*Oct  7 03:03:39.639: AAA/ACCT/3844(00000010): Pick method list 'T_ACCT_COMMANDS'

*Oct  7 03:03:39.640: AAA/ACCT/SETMLIST(00000010): Handle 6E000010, mlist 0E879058, Name T_ACCT_COMMANDS

*Oct  7 03:03:39.640: Getting session id for CMD(00000010) : db=D072908

*Oct  7 03:03:39.640: AAA/ACCT/CMD(00000010): add, count 3

*Oct  7 03:03:39.640: AAA/ACCT/EVENT/(00000010): COMMAND

*Oct  7 03:03:39.641: AAA/ACCT/CMD(00000010): Queueing record is COMMAND osr 1

*Oct  7 03:03:39.642: AAA/ACCT/CMD(00000010): free_rec, count 2

*Oct  7 03:03:39.642: AAA/ACCT/CMD(00000010): Setting session id 31 : db=D072908

*Oct  7 03:03:39.644: AAA/ACCT(00000010): Accounting method=TACACS_GROUP (TACACS+)

*Oct  7 03:03:39.645: TPLUS: Queuing AAA Accounting request 16 for processing

*Oct  7 03:03:39.646: TPLUS: processing accounting request id 16

*Oct  7 03:03:39.646: TPLUS: Sending AV task_id=30

*Oct  7 03:03:39.647: TPLUS: Sending AV timezone=UTC

*Oct  7 03:03:39.647: TPLUS: Sending AV service=shell

*Oct  7 03:03:39.647: TPLUS: Sending AV priv-lvl=15

*Oct  7 03:03:39.647: TPLUS: Sending AV cmd=no router eigrp 1 <cr>

*Oct  7 03:03:39.647: TPLUS: Accounting request created for 16(george)

*Oct  7 03:03:39.647: TPLUS: using previously set server 192.168.1.254 from group TACACS_GROUP

*Oct  7 03:03:39.649: TPLUS(00000010)/0/NB_WAIT/10FB54B8: Started 5 sec timeout

*Oct  7 03:03:39.652: TPLUS(00000010)/0/NB_WAIT: socket event 2

*Oct  7 03:03:39.653: TPLUS(00000010)/0/NB_WAIT: wrote entire 113 bytes request

*Oct  7 03:03:39.653: TPLUS(00000010)/0/READ: socket event 1

*Oct  7 03:03:39.653: TPLUS(00000010)/0/READ: Would block while reading

*Oct  7 03:03:39.657: TPLUS(00000010)/0/READ: socket event 1

*Oct  7 03:03:39.658: TPLUS(00000010)/0/READ: read entire 12 header bytes (expect 5 bytes data)

*Oct  7 03:03:39.658: TPLUS(00000010)/0/READ: socket event 1

*Oct  7 03:03:39.658: TPLUS(00000010)/0/READ: read entire 17 bytes response

*Oct  7 03:03:39.658: TPLUS(00000010)/0/10FB54B8: P

R2(config)#rocessing the reply packet

*Oct  7 03:03:39.658: TPLUS: Received accounting response with status PASS

*Oct  7 03:03:39.659: AAA/ACCT/CMD(00000010): STOP protocol reply PASS

*Oct  7 03:03:39.659: AAA/ACCT(00000010): Accounting response status = SUCCESS

*Oct  7 03:03:39.659: AAA/ACCT(00000010): Send STOP accounting notification to EM successfully

*Oct  7 03:03:39.660: AAA/ACCT/CMD(00000010): Cleaning up from Callback osr 0

*Oct  7 03:03:39.660: AAA/ACCT/CMD(00000010) Record not present

*Oct  7 03:03:39.660: /AAA/ACCTCMD(00000010) reccnt 2, csr FALSE, osr 0

Log file

root@c554333ff313:/var/log# tail -f 50 tacplus-acct.log

tail: cannot open '50' for reading: No such file or directory

==> tacplus-acct.log <==

Oct  9 08:35:45 192.168.92.254  unknown tty0    async   stop    task_id=17      timezone=UTC    service=shell   priv-lvl=15     cmd=debug tacacs <cr>

Oct  9 08:35:51 192.168.92.254  unknown tty0    async   stop    task_id=18      timezone=UTC    service=shell   priv-lvl=15     cmd=configure terminal <cr>

Oct  9 08:35:59 192.168.92.254  unknown tty0    async   stop    task_id=19      timezone=UTC    service=shell   priv-lvl=15     cmd=logging console <cr>

Oct  9 08:53:34 192.168.0.2     george  tty578  192.168.1.3     stop    task_id=24      timezone=UTC    service=shell   priv-lvl=15     cmd=show running-config <cr>

Oct  9 08:55:22 192.168.1.2     george  tty0    async   stop    task_id=22      timezone=UTC    service=shell   priv-lvl=15     cmd=do-exec sh ip int brief <cr>

Oct  9 08:55:45 192.168.92.254  unknown tty0    async   stop    task_id=20      timezone=UTC    service=shell   priv-lvl=15     cmd=do-exec sh ip int brief <cr>

Oct  9 08:57:35 192.168.0.2     george  tty0    async   start   task_id=26      timezone=UTC    service=shell

Oct  9 08:58:18 192.168.0.2     george  tty0    async   stop    task_id=26      timezone=UTC    service=shell   priv-lvl=15     cmd=debug aaa accounting <cr>

Oct  9 08:59:09 192.168.0.2     george  tty0    async   stop    task_id=27      timezone=UTC    service=shell   priv-lvl=15     cmd=debug aaa authentication <cr>

Oct  9 09:07:08 192.168.0.2     george  tty0    async   stop    task_id=28      timezone=UTC    service=shell   priv-lvl=15     cmd=configure terminal <cr>

Oct  9 09:08:27 192.168.0.2     george  tty0    async   stop    task_id=29      timezone=UTC    service=shell   priv-lvl=15     cmd=router eigrp 1 <cr>

Oct  9 09:08:54 192.168.0.2     george  tty0    async   stop    task_id=30      timezone=UTC    service=shell   priv-lvl=15     cmd=no router eigrp 1 <cr>

After applying these configurations just shut down servers and check AAA behaaviour

0.  aaa authentication login default local

1. aaa authentication login T_LOGIN_ONLY group TACACS_GROUP local

2. aaa authentication login R_LOGIN_ONLY group RADIUS_GROUP local

3. aaa authentication login R_T_LOGIN group RADIUS_GROUP group TACACS_GROUP local

4. aaa authentication login T_R_LOGIN group TACACS_GROUP group RADIUS _GROUP local

5. aaa authorization exec R_EXEC_ONLY group RADIUS_GROUP local

6. aaa authorization exec T_EXEC_ONLY group TACACS_GROUP local

7. aaa authorization exec R_T_EXEC group RADIUS_GROUP group TACACS_GROUP local

8. aaa authorization exec T_R_EXEC group TACACS_GROUP group RADIUS_GROUP local

9. aaa accounting exec R_ACCT_ONLY start-stop group RADIUS_GROUP

10. aaa accounting exec T_ACCT_ONLY start-stop group TACACS_GROUP

11. aaa accounting exec R_T_ACCT start-stop group RADIUS_GROUP group TACACS_GROUP

12. aaa accounting exec T_R_ACCT start-stop group TACACS_GROUP group RADIUS_GROUP

13. aaa accounting command 15 T_ACCT start-stop group TACACS_GROUP

|  |  | R1 | R2 | SW1 | SW2 |
| --- | --- | --- | --- | --- | --- |
| Line console 0 | LOGIN | 1 | 4 | 3 | 2 |
|  | AUTHORIZATION | 6 | 8 | 7 | 5 |
|  | ACCOUNTING EXE | 10 | 12 | 11 | 9 |
|  | ACCOUTING COMMAND | 13 | 13 | 13 | 13 |
| Line vty 0 4 | LOGIN | 2 | 3 | 4 | 1 |
|  | AUTHORIZATION | 5 | 7 | 8 | 6 |
|  | ACCOUNTING EXE | 9 | 11 | 12 | 10 |
|  | ACCOUTING COMMAND | 13 | 13 | 13 | 13 |

Compatibility matrix

| AAA Operation | Console | VTY lines | Supported by TACACS*? | Supported by RADIUS? |
| --- | --- | --- | --- | --- |
| Authentication (Login) | YES (Local / Server) | YES (Local / Server) | YES | YES |
| Authorization EXEC (Shell) | YES, recommended local/TACACS+ | YES (Server / Local) | YES (Nativ) | LIMITED (Fail often for console) |
| Authorization Commands | YES (only with TACACS+) | YES (only with TACACS+) | YES (Nativ per commmand) | NO ❌ (Does not know commands) |
| Accounting EXEC (Sessions) | YES | YES | YES | YES |
| Accounting Commands | YES (only with TACACS+) | YES (only with TACACS+) | YES (only with TACACS+) | NO ❌ (Does not know commands) |